SECRET

FUNCTION INTRODUCTION

| Function management |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function Name: |  | Comfort Stop |  |  |  |
| Function Mapping: |  | Driver assistance, autonomous driving, and chassis management |  |  |  |
| Function: |  |  |  | Function-ID: |  |
|  | Name |  | Roler | Date | signature |
| Create/Modify: |  |  |  |  |  |
| check: |  |  |  |  |  |
| Approve OMK: |  |  |  |  |  |

1	Function Introduction	4

1.1	Function description	4

1.2	Function partner list	4

1.3	Function action mode diagram	4

2	System Architecture	5

2.1	Function architecture	5

2.2	Sub-function list	6

2.3	Description of function interface	6

2.3.1	System dependency (Signals Interaction)	6

2.3.2	Hardware Interface	7

3	Basis function specification	8

3.1	Description of the function status	8

3.2	Logical signals of function	8

3.3	Base Parameters of function (calibration range)	9

3.4	State interpretation and diagram	10

3.5	Diagnostic requirements	10

3.5.1	Special Control Modes	10

3.5.2	Partial System Shutdown Modes	10

3.6	Edge conditions	10

4	Major Additional Function	10

5	Special States Detect	11

6	Functional safty	11

6.1	Hazard and Risk analysis	11

6.2	Functional safety concept	11

6.2.1	Degradation Mode	11

7	Regulatory Requirements	11

8	Function Dependencies	11

8.1	Internal	11

8.2	Extra	12

8.2.1 Self-checking strategy	12

8.2.2 Function Active Display Strategy	12

8.2.3 Fault Display Strategy	12

9	Special Performance Requirements	12

10	Appendix A	12

10.1	Change History	12

# Function Introduction

## Function description

CST(Comfort Stop), some times called PRB(Pitch Reduction Brake), The function is designed to provide the driver with a smooth feel when the car is slowing to a standstill, eliminating or minimizing the longitudinal pitch or jerk of the vehicle when it comes to a stop. It implements this by controlling the brake pressure on all four wheel.

The default status of CST switch is on when the vehicle is powered up for the first time after vehicle offline.

Figure1. Comfort Stop

## Function partner list

- iBRS

- Wheel Speed Sensor

- IMU

- LDCU

## Function action mode diagram

The function ' Comfort Stop ' calculates a request for the hydraulic brake system to control the hydraulic brake force in wheel side.

The diagrams are based on ZF IBC (Integrated Brake Controller) system

Figure2. Hydraulic Schematic

# System Architecture

## Function architecture

The CST function is designed to give the driver a smooth feeling when braking in non-emergency situations, decelerating to a standstill by applying less brake pressure than the driver requires. When the vehicle is at a standstill, the CST function increases the brake pressure to the driver's requested.

Figure 3. CST system architecture

## Sub-function list

NA.

## Description of function interface

### System dependency (Signals Interaction)

The following table lists the interactive signals for CST function. The names in the table are from VW CMP21D CEA2.0 version Comms matrix “IBRS_CEA2_CAN_MATRIX”.

| Signal Name | Sender | Tx/Rx | Cycle | Unit | Range | ASIL Level | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CDCU_CST_OFF_Switch | CDCU | RX | 100ms | Enum | 0x0: No command 0x1: ON 0x2: OFF 0x3: Reserved | QM | CST Request |
| LDCU_AccPedalSig | LDCU | Rx | 10ms | % | 0-100 | ASIL C | Accelerator pedal position signal |
| LDCU_AccPedalSigVD | LDCU | Rx | 10ms | Enum | 0x0:Not valid; 0x1:Valid; | ASIL C | LDCU_Accelerate Pedal Signal Valid |
| LDCU_GearLev | LDCU | Rx | 10ms | Enum | 0x0: Invalid; 0x1: D gear; 0x2: N gear; 0x3: P gear; 0x4:P gear; 0x5-0x7:Reserved | ASIL B | LDCU_Gear Level |
| IPB_VehLongAccel | IMU | Rx(Private) | 10ms | g | -1.73 – 1.73 | ASIL B(D) | Longitudinal Acceleration |
| IPB_VehLongAccelVD | IMU | Rx(Private) | 10ms | Enum | 0x0: Not valid; 0x1: valid; | ASIL B(D) | The valid of longitudinal acceleration |
| IPB_CST_Status | IBRS | TX |  | Enum | 0x0: CST is disabled_ Init_Default 0x1: CST is Standby 0x2: CST is Active 0x3: CST is Failure | ASIL A | CST Request Valid |
| IPB_BrkLightOnReq | IBRS | Tx | 20ms | Boolean | 0x0: Off; 0x1: On; | ASIL B | Break Light On Request |
| IPB_BrkLightOnReq_E2E | IBRS | Tx | 20ms | Boolean | 0x0: Off; 0x1: On; | ASIL B | Break Light On Request,E2E |

### Hardware Interface

| Name | Type |
| --- | --- |
| KL15 | Hardwired |
| KL30 | Hardwired |
| Left front wheel speed sensor | intelligent |
| Right front wheel speed sensor | intelligent |
| Left rear wheel speed sensor | intelligent |
| Right rear wheel speed sensor | intelligent |
| IBC CAN(H、L) | Hardwired |

# Basis function specification

## Description of the function status

In daily driving, the pitch caused by vehicle braking can easily cause users to feel uncomfortable. Under the premise of ensuring safety and sense of security, the CST function will help reduce driving fatigue and improve the comfort of passengers.

Figure 4. CST Working Schematic Diagram

## Logical signals of function

The changes condition of CST switch should meet the following criteria:

When the vehicle speed is more than 50KPH, the CST switch (IPB_CST_Status) state should be changed based on driver input.

When the vehicle speed is less than 50KPH:

- The CST switch state should be changed based on the driver input if the brake pedal is not pressed or the vehicle is standstill.

- The CST switch state should keep the current status if the brake pedal is pressed even though the driver input is changed.

The conditions of CST enable and activation should meet the following criteria:

- The enable conditions of CST function:

- The feature is not degraded by fault

And

- The CST switch is on(CDCU_CST_OFF_Switch = 0x1: On)

- The disabled conditions of CST function:

- The feature is degraded by fault

OR

- The CST switch is off(CDCU_CST_OFF_Switch = 0x2: Off)

- The activation conditions of CST function:

- CST is enable (IPB_CST_Status = 0x1: Standby)

And

- Vehicle speed is in normal range

And

- Driving direction is forward

And

- The road inclination is in normal range

And

- The brake pedal is pressed

- The deactivation conditions of CST function:

- CST is disable (IPB_CST_Status = 0x0: Off OR IPB_CST_Status = 0x3: Failure)

OR

- Vehicle speed is out of range

OR

- Driving direction is not forward

OR

- The road inclination is out of range

OR

- The brake pedal is not pressed

## Base Parameters of function (calibration range)

This table defines the required threshold parameters in the function logics. These are all base values

for a specific platform or vehicle. The thresholds are available to be tuned and verified on the test

vehicles during development.

| Conditions |  | Threshold | Remark |
| --- | --- | --- | --- |
| activation Condition | Deceleration range | -0.5m/s2＜VehDec＜-7m/s2（TBD） | Tunable |
|  | Vehicle speed | < 1.5 m/s（TBD） | Tunable |
|  | Road Inclination | < 10% | Tunable |

For a best performance expectation, the software is also required to support other variables change

Interface.

## State interpretation and diagram

The picture shows the CST subsystem work flow diagram.

## Diagnostic requirements

When there is a system malfunction that affects the ability for CST to run normal control, then the fault manager will request that CST enter a special control mode.

### Special Control Modes

- NA

### Partial System Shutdown Modes

- NA

## Edge conditions

# Major Additional Function

NA

# Special States Detect

NA

# Functional safty

## Hazard and Risk analysis

This part will be detailed in HARA. Refer to the HARA documents provided by VW.

## Functional safety concept

This part will be detailed in FSC. Refer to the FSC documents provided by VW.

### Degradation Mode

This part will be detailed in Keepalive document

# Regulatory Requirements

NA

# Function Dependencies

| External systems | Description | Responsible |
| --- | --- | --- |
| LDCU | Vehicle Info | LDCU |
| IMU | Accelaration and Yaw Single | EF |
| EPS | Steering ability | EF |
| Foundation Braking | Calipers, discs, tubing | EF |

## Internal

NA

## Extra

NA

## 8.2.1 Self-checking strategy

NA

## 8.2.2 Function Active Display Strategy

When the CST activate, need to light the brake light

## 8.2.3 Fault Display Strategy

NA

# Special Performance Requirements

Vctc provides

# Appendix A

- 《Performance Acceptance Specification》

## Change History

| Version | Change Content | Date（D/M/Y） | Reviewer |
| --- | --- | --- | --- |
| V1.0 | Initial edition | 21/07/2025 |  |